The AP55CS integrates a SiS6205 high-performance graphics and video accelerator.  It enhances the display capabilities of the system with PCI bus through its three-in-one PCI true-color graphic function.  It also enables the system to deliver a high-quality full-motion video from CD-ROM drives.

The SiS6205 utilizes the shared-memory buffer architecture.  This feature allows the display subsystem to share the system memory.

The following lists the features of the SiS6205 VGA chip:

- Enhanced 64-bit BITBLT graphics engine

- Supports 32-bit PCI rev. 2.0

- Complies with DDC1 and DDC2B specifications

- Multi-format Video for Windows support such as YUV411, YUV422, ARGB8888, RGB888, RGB656, RGB555

- Compatible with Microsoft Video for Windows

- Compatible with Real-Magic MPEG API (for interactive title)

- DCI drivers support

- 135-MHz pixel clock

- Super high-resolution graphics mode

- 640 x 480, 256/32K/64K/16M colors (NI)

- 800 x 600, 16/256/32K/64K/16M colors (NI)

- 1024 x 768, 16/256/32K/64K/16M colors (NI)

- 1280 x 1024, 16/256 colors (NI), 32K/64K colors (I)

- 2048 x 2048 maximum virtual screen

- 75-Hz maximum refresh rate

# Drivers and Utilities

## Getting Started

See to it that you have the following before you install the drivers:

- DOS 5.0, 6.0, 6.2 (or higher version) or OS/2 2.0, 2.1 (or higher version)

- VGA analog monitor

We recommend that you create backup copies of the original driver diskettes.  Store the originals and work from the backups.  If the copy gets damaged, use the original to create a new copy.  Label the working diskettes properly.

|  | Use the DISKCOPY command to create backup diskettes. Refer to your MS-DOS manual for instructions. |
| --- | --- |

## Supported Applications

The board comes with a set of display drivers for the following applications:

- 3D Studio v3.0

- AutoCAD/386 Release 11, 12

- AutoShade/386 v2.0

- GEM 3.0 /Ventura 2.0

- Lotus 1-2-3/Symphony v2.x

- Microsoft Windows 3.1

- Microsoft Windows NT v3.1 & 3.5

- Microsoft Windows 95

- OrCAD (SDT/VST/PCB) Rel. 4

- OS/2 Presentation Manager 3.0

- P-CAD v6.06

- VersaCAD/386 v2.1

- WordPerfect 5.x & 6.0

## Driver Diskettes

The AP55CS comes with a set of display drivers contained in five 1.44-MB diskettes.  The driver files contained in each diskette are as follows:

Diskette 1	Application Program Drivers

SISTAG	Disk Tag file for NT 3.1 and NT 3.5

DISK.TXT	ASCII text file describing the diskette contents

UPDATE.TXT	ASCII text file containing the driver release notes

INSTDRV.EXE	Unpack and copy program

ADI42 <DIR>	Contains the AutoCAD, AutoShade, 3D Studio driver files

GEMDRV <DIR>	Contains the GEM/Ventura driver files

LOTUS <DIR> 	Contains Lotus 1-2-3 driver files

ORCAD4 <DIR>	Contains the OrCAD driver files

PCAD6 <DIR>	Contains the PCAD driver files

UTILITY <DIR>	Contains the DOS utilities

VCAD <DIR>	Contains the VersaCAD driver files

WP51 <DIR>	Contains the WordPerfect driver files

ET24 <DIR>	Contains ETen Chinese System display 	driver (ETDSPDRV.COM) ver 1.16B01 to 
	support the SiS VGA

WINNT31 <DIR>	Contains the Windows NT 3.1 driver files

WINNT35 <DIR>	Contains the Windows NT 3.5 driver files

MANUAL <DIR>	Contains the compressed file of the SiS 6205 	user’s manual.  The file is in Winword 2.0 		format.

Diskette 2	Windows 95 and OS/2 Warp Drivers

Win95 <DIR>	Contains the Windows 95 Beta 3 driver files

OS2WARP <DIR>	Contains the IBM OS/2 3.0 (Warp) driver 	files

Diskette 3	Windows 3.1 and DCI Drivers

Root Files	SiS Windows 3.1 Driver Setup programs

SGVA <DIR>	Contains Microsoft Windows 3.1 driver files (in compressed format, default support - Windows 3.1 in U.S. version)

DCI <DIR>	Contains SiS6205 DCI driver files

Diskette 4	Microsoft Video for Windows 1.1D Runtime 	Programs

Root Files	Microsoft Video for Windows 1.1D Runtime  programs

Diskette 5	OS/2 3.0 Driver

OS23OD <DIR>	Contains the Double Bytes (Chinese, 	Japanese, Korean, etc.), IBM OS/2 3.0 	(Warp) driver files

IDEDRV <DIR>	Contains the SiS IDE driver files

## Installing the Drivers

To install the drivers, simply insert Diskette 1 into the driver diskette and type:

X:\INSTDRV	e

where X is the driver letter containing Diskette 1.  Follow the screen instructions to complete the installation.

# Standard Display Modes

The table below lists the supported standard display modes.

| MODE | TYPE | DISPLAY SIZE | COLORS SHADES | FRAME RATE | H-SYNCH | VIDEO FREQ. |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Text | 320 x 200 | 16 | 70 | 31.5K | 25.1M |
| 0* | Text | 320 x 250 | 16 | 70 | 31.5K | 25.1M |
| 0+ | Text | 360 x 400 | 16 | 70 | 31.5K | 28.3M |
| 1 | Text | 320 x 200 | 16 | 70 | 31.5K | 25.1M |
| 1* | Text | 320 x 250 | 16 | 70 | 31.5K | 25.1M |
| 1+ | Text | 360 x 400 | 16 | 70 | 31.5K | 28.3M |
| 2 | Text | 640 x 200 | 16 | 70 | 31.5K | 25.1M |
| 2* | Text | 640 x 250 | 16 | 70 | 31.5K | 25.1M |
| 2+ | Text | 720 x 400 | 16 | 70 | 31.5K | 28.3M |
| 3 | Text | 640 x 200 | 16 | 70 | 31.5K | 25.1M |
| 3* | Text | 640 x 250 | 16 | 70 | 31.5K | 25.1M |
| 3+ | Text | 720 x 400 | 16 | 70 | 31.5K | 28.3M |
| 4 | Graphics | 320 x 200 | 4 | 70 | 31.5K | 25.1M |
| 5 | Graphics | 320 x 200 | 4 | 70 | 31.5K | 25.1M |
| 6 | Graphics | 640 x 200 | 2 | 70 | 31.5K | 25.1M |
| 7 | Text | 720 x 350 | 4 | 70 | 31.5K | 28.3M |
| 7+ | Text | 720 x 400 | 4 | 70 | 31.5K | 28.3M |
| 0D | Graphics | 320 x 200 | 16 | 70 | 31.5K | 25.1M |
| 0E | Graphics | 640 x 200 | 16 | 70 | 31.5K | 25.1M |
| 0F | Graphics | 640 x 350 | 2 | 70 | 31.5K | 25.1M |
| 10 | Graphics | 640 x 350 | 16 | 70 | 31.5K | 25.1M |
| 11 | Graphics | 640 x 480 | 2 | 60 | 31.5K | 25.1M |
| 12 | Graphics | 640 x 480 | 16 | 60 | 31.5K | 25.1M |
| 13 | Graphics | 320 x 200 | 256 | 70 | 31.5K | 25.1M |

# Enhanced Video Modes

The table below lists the enhanced video modes supported by the SiS6205 PCI graphics and video accelerator.

| MODE | TYPE | DISPLAY SIZE | COLORS SHADES | FRAME RATE | H-SYNCH | VIDEO FREQ. | VGA Memory Size |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 22 | Text | 1056 x 352 | 16 | 70 | 30.5K | 40.0M | 1MB |
| 23 | Text | 1056 x 350 | 16 | 70 | 30.5K | 40.0M | 1MB |
| 24 | Text | 1056 x 364 | 16 | 70 | 30.5K | 40.0M | 1MB |
| 25 | Graphics | 640 x 480 | 16 | 60 | 31.5K | 25.1M | 1MB |
| 26 | Text | 720 x 480 | 16 | 60 | 31.5K | 25.1M | 1MB |
| 29 | Graphics | 800 x 600 | 16 | 56 | 35.1K | 30.0M | 1MB |
| 29* | Graphics | 800 x 600 | 16 | 60 | 37.9K | 40.0M | 1MB |
| 29+ | Graphics | 800 x 600 | 16 | 72 | 48.0K | 50.0M | 1MB |
| 29# | Graphics | 800 x 600 | 16 | 75 | 46.8K | 50.0M | 1MB |
| 2A | Text | 800 x 600 | 16 | 56 | 35.1K | 36.0M | 1MB |
| 2D | Graphics | 640 x 350 | 256 | 70 | 31.5K | 25.1M | 1MB |
| 2E | Graphics | 640 x 480 | 256 | 60 | 31.5K | 25.1M | 1MB |
| 2E* | Graphics | 640 x 480 | 256 | 72 | 37.9K | 31.5M | 1MB |
| 2E+ | Graphics | 640 x 480 | 256 | 75 | 37.5K | 31.5M | 1MB |
| 2F | Graphics | 640 x 400 | 256 | 70 | 31.5K | 25.1M | 1MB |
| 30 | Graphics | 800 x 600 | 256 | 56 | 35.1K | 36.0M | 1MB |
| 30* | Graphics | 800 x 600 | 256 | 60 | 37.9K | 40.0M | 1MB |
| 30+ | Graphics | 800 x 600 | 256 | 72 | 48.0K | 50.0M | 1MB |
| 30# | Graphics | 800 x 600 | 256 | 75 | 46.8K. | 50.0M | 1MB |
| 37i | Graphics | 1024 x 768 | 16 | 87 | 35.5K | 44.9M | 1MB |
| 37n | Graphics | 1024 x 768 | 16 | 60 | 48.4K | 65.0M | 1MB |
| 37n+ | Graphics | 1024 x 768 | 16 | 70 | 56.5K | 75.0M | 1MB |
| 37n# | Graphics | 1024 x 768 | 16 | 75 | 60.2K | 80.0M | 1MB |
| 38i | Graphics | 1024 x 768 | 256 | 87 | 35.5K | 44.9M | 1MB |
| 38n | Graphics | 1024 x 768 | 256 | 60 | 48.4K | 65.0M | 1MB |
| 38n+ | Graphics | 1024 x 768 | 256 | 70 | 56.5K | 75.0M | 1MB |
| 38n# | Graphics | 1024 x 768 | 256 | 75 | 60.2K | 80.0M | 1MB |
| 39i | Graphics | 1280 x 1024 | 16 | 89 | 48.8K | 80.0M | 1MB |
| 39n | Graphics | 1280 x 1024 | 16 | 60 | 65.0K | 110.0M | 1MB |
| 3Ai | Graphics | 1280 x 1024 | 256 | 89 | 48.8K | 80.0M | 2MB |
| 3An | Graphics | 1280 x 1024 | 256 | 60 | 65.0K | 110.0M | 2MB |
| 40 | Graphics | 320 x 200 | 32K | 70 | 31.5K | 25.1M | 1MB |
| 41 | Graphics | 320 x 200 | 64K | 70 | 31.5K | 25.1M | 1MB |
| 42 | Graphics | 320 x 200 | 16.8M | 70 | 31.5K | 25.1M | 1MB |
| 43 | Graphics | 640 x 480 | 32K | 60 | 31.5K | 25.1M | 1MB |
| 43* | Graphics | 640 x 480 | 32K | 72 | 37.9K | 31.5M | 1MB |
| 43+ | Graphics | 640 x 480 | 32K | 75 | 37.5K | 31.5M | 1MB |

“i”	interlaced mode

“N”	non-interlaced mode

Enhanced Video Modes (continued)

| MODE | TYPE | DISPLAY SIZE | COLORS SHADES | FRAME RATE | H-SYNCH | VIDEO FREQ. | VGA Memory Size |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 44 | Graphics | 640 x 480 | 64K | 60 | 31.5K | 25.1M | 1 MB |
| 44* | Graphics | 640 x 480 | 64K | 72 | 37.9K | 31.5M | 1MB |
| 44+ | Graphics | 640 x 480 | 64K | 75 | 37.5K | 31.5M | 1MB |
| 45 | Graphics | 640 x 480 | 16.8M | 60 | 31.5K | 25.1M | 1MB |
| 45* | Graphics | 640 x 480 | 16.8M | 72 | 37.9K | 31.5M | 2MB |
| 45+ | Graphics | 640 x 480 | 16.8M | 75 | 37.5K | 31.5M | 2MB |
| 46 | Graphics | 800 x 600 | 32K | 56 | 35.1K | 36.0M | 1MB |
| 46* | Graphics | 800 x 600 | 32K | 60 | 37.9K | 40.0M | 1MB |
| 46+ | Graphics | 800 x 600 | 32K | 72 | 48.0K | 50.0M | 2MB |
| 46# | Graphics | 800 x 600 | 32K | 75 | 46.8K | 50.0M | 2MB |
| 47 | Graphics | 800 x 600 | 64K | 56 | 35.1K | 36.0M | 1MB |
| 47* | Graphics | 800 x 600 | 64K | 60 | 37.9K | 40.0M | 1MB |
| 47+ | Graphics | 800 x 600 | 64K | 72 | 48.0K | 50.0M | 2MB |
| 47# | Graphics | 800 x 600 | 64K | 75 | 46.8K | 50.0M | 2MB |
| 48 | Graphics | 800 x 600 | 16.8M | 56 | 35.1K | 36.0M | 2MB |
| 48* | Graphics | 800 x 600 | 16.8M | 60 | 37.9K | 40.0M | 2MB |
| 48+ | Graphics | 800 x 600 | 16.8M | 72 | 48.0K | 50.0M | 2MB |
| 48# | Graphics | 800 x 600 | 16.8M | 75 | 46.8K | 50.0M | 2MB |
| 49i | Graphics | 1024 x 768 | 32K | 87 | 35.5K | 44.9M | 2MB |
| 49n | Graphics | 1024 x 768 | 32K | 60 | 48.4K | 65.0M | 2MB |
| 49n+ | Graphics | 1024 x 768 | 32K | 72 | 56.5K | 75.0M | 2MB |
| 49n# | Graphics | 1024 x 768 | 32K | 75 | 60.2K | 80.0M | 2MB |
| 4Ai | Graphics | 1024 x 768 | 64K | 87 | 35.5K | 44.9M | 2MB |
| 4An | Graphics | 1024 x 768 | 64K | 60 | 48.4K | 65.0M | 2MB |
| 4An+ | Graphics | 1024 x 768 | 64K | 72 | 56.5K | 75.0M | 2MB |
| 4An# | Graphics | 1024 x 768 | 64K | 75 | 60.2K | 80.0M | 2MB |

“i”	interlaced mode

“N”	non-interlaced mode

|  | The 45*, 45+, 46+, 46#, 47+ and 47# modes are not supported in the 1-MB DRAM configuration. |
| --- | --- |

# MPEG Function

The onboard VGA chip also supports the MPEG video playback function.  The following MPEG group icons appear in your VGA program group:

However, you need to obtain and install your own Xing MPEG drivers to enable the function.  The MPEG drivers are not included in the driver diskettes that come with the system board.