# CHAPTER 10	TOOLS§

## 1.10	PATCH EDITOR

MAKEPAT is an easy to use patch editing tool designed for use with all Aria systems.  The audio board's external MIDI interface is supported and allows interactive playback of sound while editing a patch file.

USAGE

MAKEPAT <patch>

where <patch> is an existing patch file name (.PAT will be assumed).  A DOS drive letter and path name may also be included.

Example:	Type -    MAKEPAT PATCH1

This will load PATCH1.PAT and all associated wave definitions.  If a MIDI keyboard is attached, hitting the keys will play the patch.  Otherwise, the computer keyboard can be used to preview the sound.

SCREEN LAYOUT

µ §

The Patch Edit Window is the first data screen and displays global patch parameters.

The Operator Edit Window displays operator parameters and draws the amplitude envelope shape.  Enter the Operator Window from the Patch Edit Window by hitting "Enter" when positioned on an Operator ID parameter.  "Esc" will return to the Patch Edit Window.

The Wave Definition Window displays wave definition parameters and draws the waveform.  Enter the Wave Window from the Operator Edit Window by hitting "Enter" when positioned on the Wave Table parameter.  "Esc" will return to the Operator Edit Window.

LOADING A PATCH

To load a new patch file once the editor has been started, type "ALT" E.  If the current patch has been edited, you will be prompted to save it.

A scrolling directory list of all .PAT files will be displayed.  Use the up and down arrows keys to position the highlight on the desired patch file name.  Drive letters and other directories are displayed and may also be chosen.  Hit "Enter" to load the new patch file or "Esc" to abort and return to editing the previous patch.

EDITING PROCEDURES

Changing a Parameter Value

To change a parameter value, position the highlight on the desired parameter using the directional arrow keys.  The "+" and "-" keys will increment and decrement the value by one.  The "]" and "[" keys will increment and decrement the value by ten.  Hitting "Enter" will prompt you to type a new value.

Pressing "Alt" H will cause a context sensitive help screen to appear, defining the current parameter and the range of legal values.

Editing Patch Parameters

The first screen displayed upon entering the patch editor is the Patch Edit window.  You may edit the patch name, default amplitude level, operator ID numbers, patch ID (program number), minimum number of operators, pitch wheel range, LFOs, alternate controllers, and the default pan setting.

The total number of operators is updated automatically.  Patch IDs 0 to 127 are reserved for melodic instruments, patch IDs 128 to 255 are reserved for percussive key-mapped instruments (keys 0 to 127).

Inserting, Deleting and Copying Operators

To insert a new operator, you must first make a copy of an existing one to act as a template.  While in the Patch Edit window, position the highlight on a non-zero Operator ID.  Hit "Alt" L to copy the operator information into a temporary buffer.  Now position the highlight to an Operator ID where you wish to insert the copied operator.  Press "Alt" I or "Ins" and the operator will be inserted just above the current highlight position.

To delete an existing operator, position the highlight on the Operator ID to be deleted.  Press "Alt" D  or "Del" and you will be prompted to confirm the deletion of the operator.  Enter 'Y' for yes or 'N' for no.

Editing Operator Parameters

To edit operator information, hit "Enter" when positioned over an Operator ID in the Patch Edit Window.  The Operator Edit window will then be displayed.

You may edit the operator ID number, detuning, transposition, velocity curve, balance (or pan), key bias, amplitude, envelope, pitch range and velocity range for the operator.  The graphical representation of the amplitude envelope is displayed (see ENVELOPE PARAMETERS for more information).

To view the next operator, press "Alt" N.  To view the previous operator, press "Alt" P.  You may return to the Patch Edit window by hitting "Esc".

Muting and Soloing Operators

To temporarily mute or cancel out the sound from a specific operator, press "Alt" M when positioned over the Operator ID in the Patch Edit window.  "Alt" M also mutes the current operator in the Operator Edit window.  Pressing "Alt" M a second time will unmute the operator.

To temporarily solo or listen to a single operator, press "Alt" S when positioned over the Operator ID in the Patch Edit window.  "Alt" S also solos the current operator in the Operator Edit window.  Pressing "Alt" S a second time will 'unsolo' the operator.

Editing Wave Definition Parameters

To edit a wave definition, press "Enter" when positioned over the Wave Table parameter in the Operator Edit window.  One set of wave definition parameters and a graphical representation of the waveform will be displayed.

You may edit the wave block number (index into ROM sound library -- see APPENDIX B, ROM WAVE LIBRARIES), the block size, block format and wave pitch range.

To view the next wave definition, press "Alt" N.  To view the previous wave definition, press "Alt" P.  You may return to the Operator Edit window by hitting "Esc".

Inserting, Deleting and Replacing Wave Definitions

To insert a new wave definition for the operator, enter the Wave Definition window and locate the wave definition where you wish to insert the new one.  Press "Alt" I or "Ins" and you will be prompted to enter a wave block number for the new wave definition.  The new wave definition will be inserted just before the currently displayed wave.

To delete an existing wave definition, position the highlight in the Wave Definition window to be deleted.  Press "Alt" D  or "Del" and you will be prompted to confirm the deletion of the wave.  Enter 'Y' for yes or 'N' for no.

Existing wave definitions may also be replaced by typing "Alt" R, when positioned in the desired Wave Definition window.  You will be prompted to enter a wave block number to replace the current wave definition.

ENVELOPE PARAMETERSXE "Envelope"§

Each stage of the 8-stage envelope on the Operator Edit window consists of an INDEX and a LENGTH value.

The INDEX determines the rate by which the amplitude will increase or decrease.  Increasing values range from 1 to 31, decreasing values range from -1 to -31.  Zero indicates constant amplitude.

The LENGTH specifies the amount of time spent in the envelope stage.  Each LENGTH unit is 1/344.5 second and total LENGTH for one stage can reach 1022 (3 seconds).

If a LENGTH of zero is specified, a GOTO is assumed.  A GOTO will "jump" immediately to the beginning of another envelope stage.  The destination stage is indicated by the INDEX value (0 to 7).

LISTENING TO A PATCH

If you have a MIDI-compatible musical keyboard, attach the MIDI OUT from the keyboard to the Aria MIDI IN connector.  Amplified speakers or headphones must be attached to the sound board.  During an editing session, you may play the keyboard and preview the Aria instrument.  Note on/off commands, key velocity, pitch bend and basic controllers are supported.

If you do not have a MIDI-compatible keyboard, the alphabetic keys on the computer keyboard will allow you to preview the sound.  The top row of keys (Q, W, E, R, ..., P) will play with high key velocity, the middle row (A, S, D, F, ..., ;) will play with moderate key velocity, and the lower row (Z, X, C, V, ..., ?) will play with low key velocity.

SAVING A PATCH

To save an edited patch, press "Alt" W.  To change the file name of the current patch before saving it, press "Alt" O and you will be prompted to enter a new file name.

KEYBOARD

| Arrows, PgUp/PgDn, Home/End | Move highlight around edit window |
| --- | --- |
| Enter Key | Selects item to edit |
|  | If positioned on Operator ID in Patch Edit Window, will switch to Operator Edit Window. |
|  | If positioned on Wave Table in Operator Edit Window, will switch to Wave Definition Window. |
| "Esc" | Will abort data entry. |
|  | If in Operator Edit Window, returns to Patch Edit Window. |
|  | If in Wave Definition Window, returns to Operator Edit Window. |
| +,- keys | Increment/decrement values by 1. |
| ],[ keys | Add or subtract 10 from values. |
| "Alt" D or Del | If positioned over Operator ID in the Patch Edit Window, delete an Operator. |
|  | If in Wave Definition Window, delete a wave definition. |
| "Alt" H | Context sensitive help window will be displayed. |
| "Alt" E | Load an existing patch file. |
| "Alt" I or Ins | If positioned over Operator ID in the Patch Edit Window, insert operator buffer. |
|  | If in Wave Definition Window, insert new wave definition. Will prompt for wave definition file name. |
| "Alt" L | Copy current operator to buffer. |
| "Alt" M | Mute/Unmute an operator. |
| "Alt" N | If in Operator Edit Window, go to next operator. |
|  | If in Wave Definition Window, go to next wave definition. |

| "Alt" O | Rename output patch file. |
| --- | --- |
| "Alt" P | If in Operator Edit Window, go to previous operator. |
|  | If in Wave Select Window, go to previous wave definition. |
| "Alt" R | If in the Wave Definition Window, replace the current wave definition. |
| "Alt" S | Solo/Unsolo an operator. |
| "Alt" W | Write patch file to disk. |
| "Alt" X | Exit the editor. |
| Q,W,E,R,T,Y,U,I,O,P (top row letters) | Turn on note, high velocity. |
| A,S,D,F,G,H,J,K,L,: (middle row letters) | Turn on note, moderate velocity. |
| Z,X,C,V,B,N,M,<,>,? (bottom row letters) | Turn on note, low velocity. |
| "Space bar" | Turn off all notes. |
| F1 | Zoom in on the Amplitude Envelope display. |
| F2 | Zoom out from the Amplitude Envelope display. |
| F3 | Shift Amplitude Envelope display to the left. |
| F4 | Shift Amplitude Envelope display to the right. |
| F5 | Toggle Amplitude Envelope display between showing stages 1 to 6 or showing stages 7 to 8 (release stages). |

## XE "Patch Editor" \r "PatchEditor"§XE "MAKEPAT.EXE" \r "PatchEditor"§

## 2.20	PATCH BANK UTILITY

The PBANK utility is used to create a patch bank file from individual synthesis patch files.  Either version 1 or version 2 patch bank file formats may be produced.  Version 2 formats are the condensed patch banks.

The patch bank that is produced by this utility may be used with Microsoft Windows 3.1 (version 1 format only), several of the included DOS utilities and any other custom application.  The patch bank file formats are detailed in Appendix A.

When invoked, the program will prompt for a patch bank file name to output.  Any eight character file name may be used, but the extension should always be ".BNK".

USAGE

PBANK <tname> /2

where	<tname> is a text file with patch file names specified, one per line
	/2 is an option that will produce a condensed patch bank format (version 2).

Example:  The text file "tname" might look something like this,

HORN.PAT

BRASS.PAT

STRINGS.PAT

FLUTE.PAT

|

XE "PBANK.EXE" \r "PBANK"§XE "Patch Bank Utility" \r "PBANK"§

## 2.30	PLAY MIDI UTILITY

The PLAYMIDI utility is used to playback a MIDI file from the DOS command line.  This program supports Standard MIDI File (SMF) formats 0 and 1.

If no patch bank file name is specified, the program will attempt to use either the "GENMIDI1.BNK" file or the "GENMIDI2.BNK" file, depending on the size of the Aria system ROM.  If the DOS enviroment variable, "ARIABANK", has been defined, the program will use the directory path specified.  To set the environment variable, add a line to your AUTOEXEC.BAT file:  SET ARIABANK=C:\ARIA.  This assumes that your GENMIDI1.BNK and GENMIDI2.BNK files are in a directory named \ARIA on drive C.

After a MIDI file has been started, the "space bar" will pause and resume the music playback.  The "PgUp" and "PgDn" keys can be used to adjust the volume level.  The "Esc" key will stop the music and exit the program.

If reverbXE "Reverb"§ has been chosen as a command line option, the "Home" and "End" keys adjust the level or amount of reverb that is applied.  Function keys "F1" to "F7" can be used to select the reverb mode.  If QSound Virtual AudioXE "QSound Virtual Audio"§ has been installed, the "-" key will turn off the QSound effect and the "+" key will turn on the QSound effect.

Source code written in Microsoft C language is provided with the Software Development Kit as a programming example (see PLAYMIDI.C).

USAGE

PLAYMIDI mfile <pbank> /R /Qdspfile /Mn /O

where	mfile is the MIDI file to play,
<pbank> is an optional patch bank file name,
/R is an option for loading reverb effects,
/Q is an option for loading QSound Virtual Audio,
 	dspfile must be the QSound DSP application (such as QSVA25.DSP)
/M is an option for selecting which MIDI channels will be played,
	If n = 0, all MIDI channels will play (Percussion on 10)
	If n = 1, only MIDI channels 13 - 16 will play (Percussion on 16)
	If n = 2 (default), only MIDI channels 1 - 10 will play (Percussion on 10)
/O is an option for also routing the MIDI data to the external MIDI output port

Example:  Play the MIDI file "2GUITARS.MID" with reverb effects.

	PLAYMIDI 2GUITARS.MID /R

## 2.40	ECHO MIDI UTILITY

The ECHOMIDI utility is used to play MIDI data received from the external MIDI input port through the Aria Synthesizer.

The ECHOMIDI utility allows for an optional patch bank file name to be specified as a command line option.  If no patch bank file is specified,  the program will attempt to use either the "GENMIDI1.BNK" file or the "GENMIDI2.BNK" file, depending on the size of the Aria system ROM.  If the DOS enviroment variable, "ARIABANK", has been defined, the program will use the directory path specified.  To set the environment variable, add a line to your AUTOEXEC.BAT file:  SET ARIABANK=C:\ARIA.  This assumes that your GENMIDI1.BNK and GENMIDI2.BNK files are in a directory named \ARIA on drive C.

When invoked, the program will initialize the MIDI In port to receive data.  The program will continue to receive MIDI data until the "Esc" key is pressed.  The /P option will enable printing of the MIDI data to the screen.

Source code written in Microsoft C language is provided with the Software Development Kit as a programming example (see ECHOMIDI.C).

USAGE

ECHOMIDI <pbank> /P

where	<pbank> is an optional patch bank file name,
/P is an option for printing MIDI data to the screen

Example:  Start ECHOMIDI using the default patch bank file and display MIDI data.

	ECHOMIDI /P

XE "ECHOMIDI.EXE" \r "ECHOMIDI"§XE "Echo MIDI Utility" \r "ECHOMIDI"§

## 2.50	PLAY WAVE UTILITY

The PLAYWAVE utility is used to play digital audio files using the Microsoft file format (.WAV).  This utility will only play back files that use supported sample rates and data formats.  The PLAYWAVE utility requires a wave file to be specified as a command line option.

The program will continue to play the wave file until a key is pressed or the entire file is played.  When the file is played, the following information will be displayed:

Sample Data Format:  PCM, A-Law, m-Law, or ADPCM

Sample Rate:  11025 Hz, 22050 Hz, or 44100 Hz

Bits per Sample:  8 or 16 bit

Stereo or Monophonic

As the file is playing, the current position (in bytes) will be displayed.

Source code written in Microsoft C language is provided with the Software Development Kit as a programming example (see PLAYWAVE.C).

USAGE

PLAYWAVE <sfile1> [sfile2] [sfile3] [sfile4]

where	<sfile1> is a sound file stored in the Microsoft Wave Format (.WAV).  Up to 4 mono or 2 stereo wave files may be played simultaneously, provided that their sample rates and data formats are the same.

Example:  Play the wave file, WHISTLE.WAV.

PLAYWAVE WHISTLE.WAV

XE "PLAYWAVE.EXE" \r "PLAYWAVE"§XE "Play Wave Utility" \r "PLAYWAVE"§

## 2.60	RECORD WAVE UTILITY

The RECWAVE utility is used to record digital audio files using the Microsoft file format (.WAV).  The program will prompt for several options before starting the recording.  The following formats and sample rates are available:

| Data Formats |  | Sample Rates |
| --- | --- | --- |
| 8-bit, mono |  | 7.875 KHz |
| 8-bit, stereo |  | 11.025 KHz |
| 16-bit, mono |  | 15.750 KHz |
| 16-bit, stereo |  | 22.050 KHz |
| 4-bit ADPCM mono |  | 31.500 KHz |
| 4-bit ADPCM stereo |  | 44.100 KHz |
| 8-bit m-Law mono* |  |  |
| 8-bit m-Law stereo* |  |  |
| 8-bit A-Law mono* |  |  |
| 8-bit A-Law stereo* |  |  |

*These formats are not supported by the SC18025 DSP chip

The RECWAVE utility samples signals from either an external microphone, an external line input or an auxiliary line input (such as CD-ROM audio).  The inputs may be monitored while recording.

Once a recording is started, the current position in the record buffer will be displayed.  Recording will continue until any key is pressed.  This will end the recording, save the data to a file and exit the program.

Source code written in Microsoft C language is provided with the Software Development Kit as a programming example (see RECWAVE.C).

USAGE

RECWAVE <sfile>

where	<sfile> is the name of the wave file you wish to record.

Example:  Record the wave file, TEST.WAV.

RECWAVE TEST.WAV

You will be prompted to enter the sampling source, format, rate and recording level prior to beginning the recording.

XE "RECWAVE.EXE" \r "RECWAVE"§XE "Record Wave Utility" \r "RECWAVE"§

## 2.70	ARIA BOARD TEST UTILITY

The ARIATEST utility is used to display the current configuration mapping and capability of the Aria sound card.  This provides a quick test to determine if the board is functioning correctly.

The Aria sound card's base I/O port and interrupt are displayed along with detailed system information.

Source code written in Microsoft C language is provided with the Software Development Kit as a programming example (see ARIATEST.C).

USAGE

ARIATEST

Example:  Below is a sample screen which ARIATEST may produce.

ARIATEST - Aria Board Test Program (Version 2.3)

Copyright (c) 1992-93 Sierra Semiconductor Corp.

Aria port no.   = 290h

Aria interrupt  = IRQ 10

DSP part no.    = SC18025

DSP version no. = 1.00

ROM part no.    = SC18059

ROM version no. = 1.00

ROM size        = 1024 Kbytes

ROM width       = 8 bit

Mixer type      = SC18075

RAM size        = 40 Kwords

ADC resolution  = 16 bit

Phone interface = No

DSP Applications Available in ROM:

Sound Blaster(TM) emulation

Aria PCM audio recording and playback

Aria Synthesizer(TM)

Aria Listener(TM)

Reverb effectsXE "Aria Board Test Utility" \r "ARIATEST"§XE "ARIATEST.EXE" \r "ARIATEST"§

XE "Tools" \r "Tools"§XE "Development Tools" \r "Tools"§